BrainVoyager Tutorial

Nonie Finlayson (nonie.j@gmail.com), 10/3/2014

File preparation

This section describes the file and folder organization required for pre-processing.

- Create a new folder for the participant from the template

- Download the participant files from the pacsone server

  - https://pacs.psy.ohio-state.edu/pacsone/

  - Login – ID=GolombLab – Pass=GolombLab4

  - Takes some time, downloaded as one large .zip file.

  - The files should be unzipped into the ‘DICOMS’ folder in the participant folder. The DICOMS are the raw fMRI data.

- Put all the prt (protocol) files into the participant ‘prt’ folder, and also into their respective run folders

  - Ie. expID_subjectX_funcRunX into ‘funcRunX’ folder.

  - These files describe the order of each condition

  - Scripted (xyz_data_prep)

- Open BrainVoyager – tab ‘File’ – ‘Rename DICOMS files’ – Browse to the subject DICOMS folder – ‘GO’

  - Everything is in the format ‘Patient#-block#-volume#-image#

- Then delete any unnamed files from the ‘DICOMS’ folder

  - Sometimes these unnamed files cause problems down the line.

- Copy experimental data into subject folder, then put folder in ‘Data’

- Copy over LOC and MT localizer prts to prt folder

  - funcLocObjScram

  - dat_raw

- Create _TR prts

  - LOC – run ‘make_SDMs.m’

  - MT – run ‘MTloc_makeprts.m’

Pre-processing

This section describes the steps required to pre-process the brain images. This includes creating a VMR file for the anatomical image, and FMRs and VTCs for each function run and localizers.

VMR

This is the Volume Magnetic Resonance file - creates a highres anatomical 3D image.

- Click ‘Project Wizard’ – ‘Next’ – ‘VMR project’ – ‘DICOMS’

- Label ‘subjectnumber_highres’

- Data located in the subject ‘DICOMS’ folder – find the MPRAGE file

  - This is usually ~176 files, a VMR, and early on (~5th)

- ‘OK - ‘Next’ – ‘Next’ – Change target folder to ‘subjectnumber_highres’ – ‘Finish’

  - Generally don’t need to adjust contrast/brightness

FMR

This is a Functional Magnetic Resonance file – creates a functional map for each functional run. Repeat for all functional runs.

Using xyz_scriptbv_preproc_NF.m to do FMR and motion correction

- Click ‘Project Wizard’ – ‘Next’ – ‘FMR project’ – ‘DICOMS’

- Label ‘subjectnumber_funcRunX’

  - Replace ‘funcRunX’ with the name or number of the functional run – ie. funcRun4 or MTLocalizer.

- Data located in the subject ‘DICOMS’ folder – find the required functional file

  - This information should have been recorded during the fMRI scan. Can also check on the pacsone server as well.

- ‘OK’ – ‘Next’ – ‘Next’ – change ‘Number of volumes to skip’ to 0 - ‘Next’

- Change target folder to ‘subjectnumber_funcRunX’

Create Initial and Fine Alignment

Start with highres.vmr (before aligning to old mprage, if needed)

- Tab ‘Volume’ – ‘3D volume tools’ – ‘Full dialogue’ – tab ‘Coregistration’

- ‘Select FMR’ and select the Unprocessed first functional run

  - If the slices were changed at any point in the scan, create IA and FA for that point also.

- Click ‘Align’ – ‘Run IA’ – then ‘Run FA’ if all looking good

  - This aligns the first functional run activation to the anatomical highres image, and the rest of the functional runs then use these same alignments to be consistent.

  - Only run FA if the IA looks ok – otherwise you may need to manually adjust the position of the IA first.

- To manually adjust IA, change the translation and rotation parameters in the coregistration tab, then run FA.

  - From BV manual: Often you only need to move the FMR up (negative Z translation) and maybe in the posterior direction (positive Y). Only in the case the slices were scanned under an angle would you need to change the rotation (e.g. positive X rotation).

Motion Correction

This section describes how to do manual correction for participant motion during scans.

Matlab script: The xyz_scriptbv_preproc.m script will do motion correction on each FMR.

xyz_scriptbv_preproc_NF.m

- Start with the .fmr you want to motion correct

- Tab ‘Analysis’ – ‘FMR Data Preprocessing’ – ‘Advanced’

- In ‘3D motion correction’ – ‘Options’

  - For the first run can leave this as is

  - For following runs, in ‘Reference volume..’ – ‘…’ – select ‘funcRun1.fmr’ – ‘OK’

    - Link to the unprocessed file.

- ‘GO’

  - Once the motion correction has finished, check motion. Make sure there are no sudden jumps of greater than 0.5 to 1mm.

  - The first run should start from zero, and all following runs start from similar positions than run1 finished, allowing for participant head movement between runs.

VTCs

This section describes how to create the .vtc files for each run, which are used in most of the analyses, as this links the functional activation to the anatomical highres image. Repeat this step for each functional file.

Matlab script: The xyz_scriptbv_makevtc_TAL_NF.m script will create vtc files for all subjects. Ensure you do this for all runs, including localizers.

- Start with the highres.vmr

- Tab ‘Analysis’ – ‘Create 3D-Aligned Time Course (VTC) Data’

- FMR ‘funcRunX.fmr’ – Processed version

- IA ‘funcRun1_..._IA.trf’ – FA ‘funcRun1_..._FA.trf’

  - If you want to do this in TAL space, so that these VTCs can later be linked to the flattened anatomical highres, then also:

  - Click ‘To TAL’ at top

  - AC-PC translation/rotation file – in highres folder – ‘_ACPC.trf’

  - Cerebrum border file for scaling – in highres folder – ‘_ACPC.tal’

- ‘Options’ – Adjust ‘Target resolution’ according to the fMRI specifications – ‘OK’ – ‘GO’

Flattening the Brain

This section describes both manual and automatic inhomogeneity correction for the white and grey matter, transformation into ACPC and Talairiach space, and inflation and flattening of the brain.

Inhomogeneity correction

Before we can inflate, cut, and flatten the brain, we need to correct inhomogeneities in the white and grey matter, to ensure correct transformation when flattening.

Start with the highres.vmr (not TAL)

Automatic (try this first)

- Tab ‘Volumes’ – ‘Inhomogeneity Correction, V16 Tools’

- Click ‘go’ in the IIHC section to start automatic correction

  - 2 runs through is fine

- Save .V16 file (_IIHC for automatic correction, _hom for homogeneity version)

Check separation of grey/white matter

- Open ‘3D Volume Tools’ (Full Dialogue) and go to the ‘Segmentation’ tab

- Change Min. Value Range to something small (~50), then click ‘Grow Region’

  - Trying to select the whole brain. This requires the brain to be extracted from the skull.

- Tab ‘Volumes’ – ‘Intensity Histogram’

  - This will display a graph, hopefully showing two peaks – one for grey, one for white matter.

Extract brain from skull

- Tab ‘Volumes’ – ‘Inhomogeneity Correction, V16 Tools’

- Under ‘Spatial Transfigurations’ click ‘Extract Brain’

Manual (try this if automatic inhomogeneity correction isn’t sufficiently homogenizing the white matter)

- Open ‘3D Volume Tools’ (Full Dialogue) and go to the ‘Segmentation’ tab

- Hover mouse over a section of white matter in the brain, and check the intensity

  - Displayed in the ‘Info’ window on the right

- Change the Value Range Min. to this intensity

  - Usually between ~120-160. Don’t start too low.

- Ensure Max. is 255

- Click ‘Grow Range’

  - This should highlight some of the white matter

- Reduce the Min., and click ‘Extend’

  - This will increase the blue selected region to any sections of that intensity, without selecting completely new regions. Repeat this last step until sufficient white matter is selection

- If you select too much, click ‘Reload All’

  - This will reload the most recently saved .V16 to work with

- Tab ‘Volumes’ – ‘Inhomogeneity Correction, V16 Tools’

- Click ‘Correct IIH’

  - This should reduce the inhomogeneity within the selected white matter.

- Save .V16 (_hom) and .vmr (_hom)

  - At this point, you can either check the grey/white matter separation again, or redo the manual inhomogeneity correction multiple times.

  - Can also try to adjust the brightness/contrast from within the ‘Inhomogeneity Correction, V16 Tools’. This may improve W/G matter separation.

Brain Transformation

Start with the .vmr file you want to transform

AC-PC transformation

Try automatic first

- Open ‘3D Volume Tools’ (Full Dialogue) and go to the ‘Talairach’ tab

- Click ‘Detect MSP, AC, PC’

  - Bottom images are the detected locations. Check to see if the AC points is correctly pin-pointed. If so, click ‘Transform to ACPC’. If not, try manual.

- Click ‘Manual’ – ‘Find AC point…’

  - Use the arrow keys to move around the images to locate the AC point. Holding Shift+up/down will move forward/backward.

- Click ‘OK’ once AC point located

- Click ‘Find AC-PC Plane’

  - This time best to use the adjust buttons provided to adjust the images until in the correct position

- Click ‘OK’ once in the correct plane in all three dimensions

- Click ‘Transform to ACPC’

Talairach transformation

- Open ‘3D Volume Tools’ (Full Dialogue) and go to the ‘Talairach’ tab

- Click ‘Auto-detect’

  - Visually check the frame that is created around the brain

- In the box below ‘Auto-Detect’, select each point in turn to ensure it is correctly placed. Click ‘Show point’ to see each selected point, and click ‘Set point’ as you adjust each point.

  - AC – anterior commissure, PC – posterior commissure, AP – most anterior point on the brain, PP – posterior point, SP – superior point, IP – inferior point, RP – right point, LP – left point. Remember that the brain is usually flipped horizontally here.

- Save .TAL, then click ‘ACPC  TAL’, and ‘GO’

Segmentation

Once you’re happy with the homogeneity corrections and the brain has been transformed into TAL space, segment the brain.

- Open ‘3D Volume Tools’ (Full Dialogue) and go to the ‘Segmentation’ tab

- Click ‘Autom. Segm.’

- Click ‘GO’

  - Defaults are fine. In the second run through, check the ‘Remove bridges LH/RH’.

- Two graphs should appear – ‘Smoothed intensity histogram’ and ‘Grey and White matter peaks…’

  - The G/W matter peaks graph should show the blue line above the red line between 20-80 (as much as possible). If not, re-do Inhomogeneity correction.

  - If the G/W matter peaks graph looks ok, check the location of the cut-off on the intensity histogram. Trying to maximize difference between G/W matter, so adjust if necessary.

Re-doing transformation after re-doing inhomogeneity correction

You don’t need to go through all the same steps for the ACPC and TAL transformations more than once. If you needed to go back to the .V16 and .vmr, you can follow these steps for a quick re-transformation.

- Open ‘3D Volume Tools’ (Full Dialogue) and go to the ‘Spatial Transf’ tab

- Click ‘Load .TRF’ – open _ACPC.trf

- Click ‘Transform .VMR’ – ‘GO’

- Open ‘3D Volume Tools’ (Full Dialogue) and go to the ‘Talairach’ tab

- Click ‘Load .TAL’ – open ‘_ACPC.tal’

- Click ‘ACPC  .TAL’ – ‘GO’

- Then re-do segmentation

Flattening the Brain

We do each hemisphere separately, so each step below should be repeated for the right and left hemisphere.

Check the segmentation

- Open ‘_BL2.vmr’

- Tab ‘File’ – ‘Load secondary vmr’ – open ‘_hom_TRF_TAL.vmr’

- F9 tabs between overlay, F8 turns the secondary .vmr on/off

  - Scroll through the brain here to check how the segmentation of the white and grey matter went.

- Tab ‘Meshes’ – load mesh – ‘_RECOSM.srf’

  - This loads the smooth reco file. To move around brain, use the mouse for rotation. Shift+mouse enables lateral movements, and Ctrl+shift+mouse enables zooming.

- In left ‘Info’ tab, click grey head with arrow ‘Select Vertex Mode’

  - This locks the brain viewpoint, and links it with the .vmr, so that you can compare points. Can also go to tab ‘Window’ – ‘Tile’ to better compare images.

Inflate & flatten brain

- Tab ‘Meshes’ – ‘Mesh Morphing’

- Check ‘Inflation Mode’ – change ‘Number of Iterations’ to either 1000, or do twice at 500, and check ‘Update curvature coloring during morphing’ – ‘GO’

  - Check it is referencing the correct file (_RECOSM.srf)

  - Brain should inflate. If it blows up or doesn’t work, you likely need to re-do the inhomogeneity correction and segmentation.

- Tab ‘Meshes’ – Save mesh as ‘_INFL’

- In left ‘Info’ tab, click red head with arrow ‘Draw/cut Mesh Mode’

  - This will allow us to draw a line matching the Calcarine manually for the next step.

- Hold down shift and click five times along the Calcarine to draw a line

  - If you make a mistake and need to reload the mesh – tab ‘Meshes’ – load mesh ‘_INFL’.

- Tab ‘Meshes’ – ‘Mesh Morphing’ - Click ‘Auto cut’ – check ‘Use specified calcarine cut’ – ‘Apply’

  - Here you can quit out of the ‘Mesh’ window to check the cuts.

- Check ‘Unfolding mode’ – ‘GO’

  - Brain should cut and flatten. If this explodes, need to go back to inhomogeneity and separation steps.

- Tab ‘Meshes’ – ‘Mesh Morphing’ – Check ‘Distortion correction’ – ‘GO’

  - This will smooth out the edges that have been distorted during the unfolding because they were wrapped around etc.

- Tab ‘Meshes’ – ‘Background and Curvature color’

  - Here we change the coloring of the flattened brain.

- Click ‘Curvature’ after each of the next step to make change

- Click ‘Use two colors for convex and concave’

- Change ‘Default and convex color’ to light grey and ‘Concave curvature color’ to darker grey. Click ‘Smooth’ once - ‘OK’

- Tab ‘Meshes’ – save mesh as ‘_flat’

Analyses

This section describes various analyses you can perform in BrainVoyager.

GLM

General Linear Model – this compares the activation in one set of voxels to the activation in another set of voxels.

Single run

- Start with highres.vmr

  - TAL.vmr if now working in TAL space

- Tab ‘Analysis’ – ‘Link 3D Time Course (VTC) File’ - Browse to run for analysis – ‘OK’

  - Ie. funcRun1_..._TAL.vtc

- Tab ‘Analysis’ – ‘General Linear Model: Single Study…’ – Load .sdm – ‘GO’

  - Make sure you’re in the right run folder, and choose the correct file (ie. TR)

- Tab ‘Analysis’ – ‘Overlay GLM’ – choose contrast you want to view – ‘OK’

  - Click ‘Balance +/-‘ if choosing an unbalanced contrast

  - Can ‘Save .CTR’ to save the contrast – if you have one saved, click ‘Load .CTR’

    - This saves having to check the correct boxes each time

  - Can save as a .vmp (volume map) – Analysis – overlay volume maps – change ‘map name’ to s0X_lvr/uvd etc, then save as (in MeanGLM_VMPs_tal).

Multiple runs

- Start with highres_TAL.vmr

  - You will create .MDMs first, and then use these to run the .GLMs.

- Tab ‘Analysis’ – ‘General Linear Model: Multi Study…’

- From here, either load a saved .MDM or ‘Add to list’ to create an .MDM, and then save it.

  - Once you have one .MDM made, you can open these files in a text editor to create more. This is quicker than creating through BV.

- Name .GLM in ‘Resulting GLM file’

  - This is where and what the .GLM will save as.

- Tab ‘Analysis’ – ‘Overlay GLM’ – choose contrast you want to view – ‘OK’

  - Click ‘Balance +/-‘ if choosing an unbalanced contrast

  - Can ‘Save .CTR’ to save the contrast – if you have one saved, click ‘Load .CTR’

  - Once you have run and saved the .GLM in the previous step, you can load it from here, and then run the required contrasts, directly from the .vmr.

Across participants

- Make one .MDM for all participants

  - Copy the .MDMs from each participant into one file, making sure to keep the ‘NrOfStudies’ correct.

- Open a TAL.vmr

- Tab ‘Analysis’ – ‘General Linear Model: Multi Study…’

- Load .MDM

- Name .GLM in ‘Resulting GLM file’

- Tab ‘Analysis’ – ‘Overlay GLM’ – choose contrast you want to view – ‘OK’

- Tab ‘Analysis’ – ‘Overlay Volume Maps’ – Tab ‘Map Options’

- Change P-value

  - Usually want p < .001

- Set Cluster threshold (in Statistics) – change voxels (decide later, eg. >50)

- Tab ‘Browse’ – Save As…

  - You can also adjust the threshold levels using the buttons on the left of the screen with yellow patches – ‘Decrease Threshold’ and ‘Increase Threshold’

RetMap ROIs

This section describes the art behind choosing ROI patches from the retinotopic mapping data. Need to run retMaps_analyze_vtcs.m to create the .vmp files first. This collates the data from the various retMap runs completed. Need to do these separately for LH and RH.

Create retinotopic heat map in each hemisphere

Start with TAL.vmr

- Tab ‘Analysis’ – ‘Overlay Volume Map’ – Load  - .vmp average retMaps

  - Using the mirror vs. standard file just depends on which creates the best color map for distinguishing between areas. Found that mirror is good for RH, standard for LH.

  - Warning – this may look like a solid colored box over the brain – this is ok.

- Tab ‘Meshes’ – ‘Load Mesh’

  - Usually in highres - load INFL_flat_L/RH.srf

- Tab ‘Meshes’ – ‘ Surface Maps’ – Create SMP – ‘OK’

- Tab ‘Statistics’ – ‘Lag Values’

  - Here, you can also adjust the default LUT to use.

- Tab ‘Advanced’ – can try ‘Smooth SMP’ but don’t use much

- Adjust threshold (between 0.3-0.4) – Update SRF

- Save as.. s0X_retMap_L/RH.smp – in main participant folder

Load various data

Load TAL.vmr – load mesh – load surface map

- In .vmr tab – tab ‘Analysis’ – ‘Overlay Volume Maps’ – Load

  - Here there are many options for the data you can choose to view while trying to choose ROIs. The above step has already demonstrated how to load the retinotopic mapping data, but you may also have other localizers you want to view (ie. MT and LO localizers), as well as overlaying your trial data to see which areas are being used in your experiment – but don’t use this to define the maps as it is circular.

  - E.g. Load the mean-x GLM maps. Ie. This participant’s MeanGLM_VMPs_tal in the main study folder.

- In .mesh tab – tab ‘Meshes’ – ‘Surface Maps’ – ‘Create SMP’ – ‘OK’

- With new map selected - Tab ‘Statistics’ – ‘Use SMP colors’

  - You can adjust these colors so that they are viewable underneath the retMap.

- Tab ‘Advanced’ – ‘Multiple selections’ and ‘Apply Immediately’

- ‘Save as’ when you have the SMPs you want to use for mapping.

- Tab ‘Browse’ – highlight the retmap for retinotopic mapping

  - You will pick the visual areas based on the changing of colors on the retMap. This is the more-art-less-science part, so if you don’t know how to do this, ask someone to help.

Select patches-of-interest

- Click ‘Draw/Cut Mesh Mode’ – hold Shift and click around the patch/s you want to select.

- Hold mouse ‘Right click’ – Save Marked Region

- Click ‘Fill Mode’ – click in selected patch – hold ‘Right Click’ – Define POI – Name patch

  - You can use either the visual area naming convention, or the brain region naming convention. Ie. s0X_V1_LU – visual area 1, left upper. OR s0X_V1v_RH

  - Sometimes the first patch is the same color as the cuts, so you should click somewhere unwanted and name it temporarily, and delete later.

  - You can also highlight other patches to help with choosing each area.

- Once all the POIs have been selected, in ‘POI Analysis’ – Save as.

  - If this window has been closed, you can open it again in Tab ‘Meshes’ – Patch-of-Interest Analysis’.

- Highlight all POIs – ‘Options’ – ‘POI Functions’ – in ‘POIs  VOIs:…’ click ‘Create’

  - This transforms the patches to volume space. This opens the VOI Analysis window – which can be reopened in ‘Analysis’ – ‘Region-of-Interest Analysis’.

- Save all the VOIs once created

- From the VOI window you can create combined VOIs

  - Ie. highlight all V1_xx VOIs - click ‘a OR b’ – name as s0X_V1.

Create LOC VOIs

This section describes how to create LOC volumes of interest, and can be used to create other localizer VOIs. These steps should be replicated for the LOC in each hemisphere.

Start with TAL.vmr

- Tab ‘Analysis’ – ‘Overlay GLM’

- Load the functional localizer – set/load contrasts

  - Ie. for LOC – funcLocAll

  - For LOC – Obj vs. Scram

  - For higher parietal areas – Fix vs. Obj. & Scram

- Adjust threshold to see more/less activation

- Tab ‘Volumes’ – ‘3D Volume Tools’ – ‘Full Dialog’ – Change Range ‘~25’

  - This sets the range of voxels that are selected when you click on the localizer activation.

- Choose LOC and click on it, selecting an area – right click – ‘Show ROI Time Course’

- Click on graph to open more options – ‘Options’ – ‘Define VOI’ – Name VOI

  - This should add the new VOI to the ‘VOI Analysis’ window. Once all localizer maps are chosen, make sure you re-save the VOI list.